Expt 7.1 Spectral Estimation

Program:

clc; clear; close all fs=1000; t=(0:999)/fs; s=sin(2*pi*100*t)+0.5*sin(2*pi*200*t); n=0.3*randn(size(t)); x=s+n; figure, plot(x), grid on title('Signal with Noise') figure, periodogram(x,[],[],fs) title('PSD using Periodogram') figure X=abs(fft(x)); f=(0:length(X)-1)*fs/length(X); plot(f(1:500),X(1:500)), grid on title('Magnitude Spectrum') figure bar([mean(s.^2) var(n)]) set(gca,'XTickLabel',{'Signal','Noise'}) title('Power Comparison') grid on

Output:

7.2

Program:

clc; clear; close all; % Signal fs = 1000; N = 256; t = (0:N-1)/fs; x = sin(2*pi*100*t) + 0.8*sin(2*pi*110*t);

NFFT = 2048; f = (0:NFFT/2-1)*(fs/NFFT);

% Windows w = {rectwin(N), hamming(N), hann(N)}; names = {'Rectangular','Hamming','Hanning'};

% Processing

for i = 1:3     xw = x(:).*w{i};

X = fft(xw,NFFT);

P{i} = abs(X(1:NFFT/2));     P{i} = P{i}/max(P{i});

end

%% Individual Spectrums

figure; for i = 1:3     subplot(3,1,i);     plot(f,20*log10(P{i}),'LineWidth',1.5);     title(['Spectrum using ' names{i} ' Window']);     xlim([0 250]); grid on; end %% Comparison

figure; plot(f,20*log10(P{1}),'k',f,20*log10(P{2}),'r',f,20*log10(P{3}),'b','LineWidth',1.5); legend(names); title('Window Comparison'); xlim([50 170]); grid on;

%% Window Shapes

figure; for i = 1:3     subplot(3,1,i); plot(w{i},'LineWidth',1.5);     title([names{i} ' Window']); grid on; end

%% Peak Detection

for i = 1:3

[pks{i},loc{i}] = findpeaks(P{i},f,'MinPeakHeight',0.3);     fprintf('\n%s Window Frequencies:\n',names{i});

disp(loc{i});

end

%% Peak Marking (Hamming)

figure; plot(f,20*log10(P{2}),'LineWidth',1.5); hold on; plot(loc{2},20*log10(pks{2}),'ro','MarkerFaceColor','r'); title('Detected Frequencies (Hamming)'); xlim([50 170]); grid on;

%% Main Peaks fprintf('\nMain Spectral Peaks:\n'); for i = 1:3

[~,idx] = max(P{i});     fprintf('%s Window Peak = %.2f Hz\n',names{i},f(idx)); end

Output: