C03 ASSESSMENT TOOL 2

25. Audio Signal Sampling Rate Conversion

Simulate sampling rate conversion of an audio signal from 44.1 kHz to 48 kHz and evaluate

its performance. (25) (BL5) (WK4)

Given Data:

● Input Sampling Rate = 44.1 kHz

● Output Sampling Rate = 48 kHz

Tasks:

1. Design the converter.

2. Perform sample rate conversion.

3. Analyze frequency response.

4. Compare audio spectra.

5. Evaluate distortion.

6. Discuss implementation

Introduction

smapling rate conversion (SRC) is a digital signal processing technique used to change the sampling frequency of a digital audio signal while preserving its original quality. It is widely used in audio processing, multimedia applications, digital communication systems, and audio playback devices where different systems operate at different sampling rates. In this experiment, an audio signal sampled at 44.1 kHz is converted to 48 kHz, which is a common requirement when transferring audio between CD-quality systems and professional audio equipment.

program

# clear;

# close all;

# Read input audio

# [x, Fs] = audioread('audio.wav');

# Fs_new = 48000;

# % Sampling Rate Conversion

# y = resample(x, Fs_new, Fs);

# % Time vectors

# t1 = (0:length(x)-1)/Fs;

# t2 = (0:length(y)-1)/Fs_new;

# % Plot Input Signal

# figure;

# subplot(2,1,1);

# plot(t1,x);

# title('Input Audio Signal (44.1 kHz)');

# xlabel('Time (s)');

# ylabel('Amplitude');

# % Plot Output Signal

# subplot(2,1,2);

# plot(t2,y);

# title('Output Audio Signal (48 kHz)');

# xlabel('Time (s)');

# ylabel('Amplitude');

# % Frequency Spectrum

# N1 = length(x);

# N2 = length(y);

# X = abs(fft(x));

# Y = abs(fft(y));

# f1 = (0:N1-1)*Fs/N1;

# f2 = (0:N2-1)*Fs_new/N2;

# figure;

# subplot(2,1,1);

# plot(f1,X);

# title('Input Spectrum');

# xlabel('Frequency (Hz)');

# subplot(2,1,2);

# plot(f2,Y);

# title('Output Spectrum');

# xlabel('Frequency (Hz)');

# % Distortion (MSE)

# x_new = resample(y, Fs, Fs_new);

# L = min(length(x),length(x_new));

# mse = mean((x(1:L)-x_new(1:L)).^2);

# disp(['Mean Square Error = ', num2str(mse)]);

# Tool Used

# Software: MATLAB

#   Toolbox: Signal Processing Toolbox

#   Functions Used:

# audioread() – Reads the input audio file.

# resample() – Performs sampling rate conversion.

# fft() – Computes the frequency spectrum.

# plot() – Displays time and frequency domain signals.

# mean() – Calculates Mean Square Error (MSE) for distortion analysis

Result:

  The audio signal was successfully converted from 44.1 kHz to 48 kHz using the resample() function.

  The output waveform maintained the same audio content with a higher sampling rate.

  The frequency spectrum of the converted signal closely matched that of the original signal, indicating that the important frequency components were preserved.

  The calculated Mean Square Error (MSE) was very small, showing that the distortion introduced during sampling rate c

.